SP-G Flow Actuator

Exd ATEX Evaluation

Table of contents

1. General information	3

1.1 Product description	3

1.2 ATEX Template	5

2. Assembly design	7

2.1 Construction category	7

2.2 Gas Group	7

2.3 Temperature Class	7

3. Assembly details	7

4. Evaluation ATEX	8

5. Conclusion	10

6. Contact	11

1. General information

1.1 Product description

Exd ATEX SP-G series product description

The SP-G liquid flow actuator is designed to move a lever when an enough flow of liquid is reached. The function is based solely on the flow of the liquid and is not affected by the system’s static pressure; however, the system does require a basic pressure more than 1 bar.

The actuator is customized for applications where alarms for low flows are required while large flows can pass through without any increased fall in pressure.

Control unit					Pipe section.

SP-G consist of two main parts, a pipe section which contain a dimensioned pressure plate and a control unit. Pressure plate allows to choose required actuation flow value from 1.5 to 30 l/min.

The actuator has a variable through-flow area that increases as the flow increases.

The through-flowing fluid lifts a spring-loaded pressure plate. The shaft on which the plate is mounted is mechanically connected to a lever that can actuate a micro-switch. The dimension of the pressure plate controls the position of the lever angle. Lever seals between the pipe section, which is pressurized from the inside of the pipe, and the inner part of the control unit which is exposed to the atmosphere.

Those parts that encounter the liquid are manufactured in Teflon and 316L stainless steel. Synthetic rubber gaskets.

See below a sketch about function and internal parts.

SP-G15

SP-G20

SP-G15 are fitted with ½” female threaded connection and SP-G20 are fitted with ¾” male threaded connection.

In the case of the Exd ATEX version of SP-G series, the mechanical part of the flow actuator is associated to one already Exd ATEX approved microswitch.

Switch marking:             II 2G Ex d b IIC T6 Gb

All different models are according to codification table on page 5.

1.2 ATEX Template

Last part of codification, see standard leaflet.

Details about the different SP-G Exd types:

SP-G15 ATEX	 			SP-G15 NPT ATEX

SP-G20 ATEX	 			SP-G20 NPT ATEX

2. Assembly design

2.1 Construction category

Assembly is designed to be installed inside zone 1, construction category is II 2G (For surface = II, Instrument category = 2, for gas atmosphere = G)

2.2 Gas Group

Assembly is designed to be installed into Gaz atmosphere, for gas group IIC.

2.3 Temperature Class

Temperature Class

Maximum temperature which can be reached by flow actuator body is the fluid temperature which is going through. Temperature class will consequently be according to max fluid temperature. Maximum temperature accepted by flow actuator = 120°C

Temperature classes are according below table.

Reminder regarding temperature class values (temperature class value is the maximum surface temperature which can be reached by the flow actuator:

For information Standard is stating: 	T4 : 135°C	T5 : 100°C	T6 : 85°C

| Ambient Temperature | Media Temperature | Temperature Class |
| --- | --- | --- |
| Tamb < 70°C | T media <85°C | T6 |
| Tamb < 70°C | T media < 90°C | T5 |
| Tamb < 70°C | T media < 120°C | T4 |

3. Assembly details

The totally mechanical flow actuator is associated with an already ATEX certified microswitch. As the mechanical element is the one which is not certified, assembly will be considered as mechanical instrument. ATEX directive 2014/34/EU will then be applicable to this assembly.  A risk analysis has been performed for the flow actuator. The assembly will be CE marked, will get an EU conformity declaration and a specific manual.

Nonelectrical component: Flow actuator according to description on codification table on page 5.

Electrical component: Microswitch with marking             II 2G Ex d b IIC T6 Gb

4. Evaluation ATEX

Ignition risk evaluation of the flow actuator has been carried out according EN ISO 80079-36 for non-electrical equipment for explosive atmosphere. According to assessment table, it appears there is no ignition risk from the flow actuator. Flow actuator is fitted with already ATEX approved microswitch. Association of the two components is therefore not bringing any ignition risk.

*Note: Earth connection is mandatory, see manual and installation instruction.

| Possible Ignition Sources | Equipment Related Yes/No | Reason |
| --- | --- | --- |
| Hot surfaces | Yes | High temperature of measured fluid (120°C), can raise outside temperature of the pipe section to temperature close to fluid temperature. |
| Mechanical sparks | No | The energy of the moving parts is too low to cause sparks. |
| Flames, hot gases | No | Nothing inside the flow actuator can ignite the gas. |
| Stray electrical currents and cathodic corrosion protection | No | Not present / NA |
| Static electricity | Yes | Friction on external parts of the meter. * |
| Static electricity | No | Friction of the fluid with pressure plate inside pipe section. All internal parts will be filled into liquid, so there is not possibility of spark appearing |
| Frictional heat | No | The movement of the lever is too slow to cause the flexible seal to heat up. |
| Lightning | No | Not present / NA |
| Electromagnetic waves | No | Not present/ NA |
| Ionizing radiation | No | Not present / NA |
| High frequency radiation | No | Not present / NA |
| Ultrasonic | No | Not present / NA |
| Adiabatic compression | No | Not present / NA |
| Chemical reaction | No | Not present / NA |

Attached file: Risk analysis table 20230925 rev 1.xlsx

This technical file been submitted to a retention of technical documentation by the LCIE according with the relevant Chapter 3, article 13, point 1.b.ii of the Directive 2014/34/EU.

This technical file was sent for conservation to LCIE. LCIE retention file number is: ***************

5. Conclusion

SP-G flow actuator ATEX version are compliant with EPL Gb and can be marked:

II 2G Ex h d IIC T6(85°C) T5(90°C) T4(120°C) – Gb.

6. Contact

Orders/inquiries: 	+468 603 07 80

Switchboard:	+468 603 07 70

E-mail:	info@eletta.com

Web:		www.eletta.com

Address:	Eletta Flow AB

P.O Box 5084

SE-141 05 Kungens Kurva
Sweden